| Device | Interface | IP Configuration |
| --- | --- | --- |
| One | s0/3/0 | 192.168.30.1 |
| One | s0/3/1 | 192.168.20.2 |
| Two | s0/3/0 | 192.168.10.2 |
| Two | s0/3/1 | 192.168.20.1 |
| Three | s0/3/0 | 192.168.30.2 |
| Three | s0/3/1 | 192.168.10.1 |
| One | g0/0 | 192.168.100.1 |
| Two | g0/0 | 192.168.150.1 |
| Three | g0/0 | 192.168.50.1 |

#hostname One

#enable password One1

#enable secret One1

En

Conf t

No ip domain lookup

Line console 0

Password SkomplikowaneHaslo

Login

exit

Enable secret SkomplikowaneHaslo

Line vty 0 15

Password SkomplikowaneHaslo

Exit

Banner motd 'Projekt WAN'

Exit

copy running-config startup-config

Line vty 0 4

Password SkomplikowaneHaslo

Exit

Line aux 0

Password SkomplikowaneHaslo

Login

exit

One(config)#int s0/3/0

One(config-if)#ip address 192.168.30.1 255.255.255.252

One(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/3/0, changed state to down

One(config-if)#

%LINK-5-CHANGED: Interface Serial0/3/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/0, changed state to up

One(config-if)#int s0/3/1

One(config-if)#ip add

One(config-if)#ip address 192.168.20.2 255.255.255.252

One(config-if)#no shutdown

One(config-if)#

%LINK-5-CHANGED: Interface Serial0/3/1, changed state to up

One(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/1, changed state to up

Two#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Two(config)#int s0/3/0

Two(config-if)#ip add 192.168.10.2 255.255.255.252

Two(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/3/0, changed state to down

Two(config-if)#int s0/3/1

Two(config-if)#ip ad

Two(config-if)#ip address 192.168.20.1 255.255.255.252

Two(config-if)#no shutdown

Two(config-if)#

%LINK-5-CHANGED: Interface Serial0/3/1, changed state to up

Two(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/1, changed state to up

%LINK-5-CHANGED: Interface Serial0/3/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/0, changed state to up

Three(config)#int s0/3/0

Three(config-if)#ip address 192.168.30.2 255.255.255.252

Three(config-if)#no shutdown

Three(config-if)#exit

Three(config)#int s0/3/1

Three(config-if)#ip add 192.168.10.1 255.255.255.252

Three(config-if)#no shut

%LINK-5-CHANGED: Interface Serial0/3/1, changed state to down

Three(config-if)#

%LINK-5-CHANGED: Interface Serial0/3/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/1, changed state to up

Three#

%SYS-5-CONFIG_I: Configured from console by console

One#conf t

Enter configuration commands, one per line. End with CNTL/Z.

One(config)#int g0/0

One(config-if)#ip add 192.168.100.1 255.255.255.0

One(config-if)#no shut

One(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

One#

%SYS-5-CONFIG_I: Configured from console by console

Two(config-if)#int g0/0

Two(config-if)#ip add 192.168.200.1 255.255.255.0

Two(config-if)#no shut

Two(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

Three(config)#int g0/0

Three(config-if)#ip add 192.168.50.1 255.255.255.0

Three(config-if)#no shut

Three(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

BGP

EXTRA_ONE(config)#router bgp 202

EXTRA_ONE (config-router)#neig

EXTRA_ONE (config-router)#neighbor 192.168.80.1 remote-as 101

EXTRA_ONE (config-router)#%BGP-5-ADJCHANGE: neighbor 192.168.80.1 Up

EXTRA_ONE (config-router)#netw

EXTRA_ONE (config-router)#network 172.16.00.0 mask 255.255.0.0

Four(config)#router bgp 101

Four(config-router)#neig

Four(config-router)#neighbor 192.168.80.2 remo

Four(config-router)#neighbor 192.168.80.2 remote-as 202

Four(config-router)#%BGP-5-ADJCHANGE: neighbor 192.168.80.2 Up

Four(config-router)#netwo

Four(config-router)#network 192.168.200.0 mask 255.255.255.0

RIP

EXTRA_TWO(config)#router rip

EXTRA_TWO(config-router)#version 2

EXTRA_TWO(config-router)#network 192.168.90.0

EXTRA_TWO(config-router)#network 192.168.170.0

EXTRA_TWO(config-router)#auto

EXTRA_TWO(config-router)#no auto-summary

EXTRA_TWO(config-router)#exit

EXTRA_ONE(config)#router rip

EXTRA_ONE(config-router)#version 2

EXTRA_ONE(config-router)#network 192.168.90.0

EXTRA_ONE(config-router)#network 172.16.0.0

EXTRA_ONE(config-router)#no auto

EXTRA_ONE(config-router)#no auto-summary

EXTRA_ONE(config-router)#exit

EIGRP

SIX(config)#router eigrp 30

SIX(config-router)#network 192.168.70.0 0.0.0.3

SIX(config-router)#

%DUAL-5-NBRCHANGE: IP-EIGRP 30: Neighbor 192.168.70.1 (Serial0/3/0) is up: new adjacency

SIX(config-router)#network 192.168.251.0 0.0.0.255

SIX(config-router)#exit

SIX(config)#exit

ACCESS LIST

Three(config)#ip access-list sta

Three(config)#ip access-list standard ACL3

Three(config-std-nacl)#permit 192.168.150.10

Three(config-std-nacl)#permit 192.168.150.20

Three(config-std-nacl)#permit 192.168.100.20

Three(config-std-nacl)#permit 192.168.200.20

Three(config-std-nacl)#permit 192.168.100.10

Three(config-std-nacl)#exit

Three(config)#int s0/3/1

Three(config-if)#ip acc

Three(config-if)#ip access-group ACL3 in

VLAN

Vlan 50

Name TWO

s1(config)#int f0/4

s1(config-if)#swi

s1(config-if)#switchport mode ac

s1(config-if)#switchport mode access

s1(config-if)#swit

s1(config-if)#switchport ac

s1(config-if)#switchport access vlan 50

s1(config-if)#end